SANDISK INTERNAL STRATEGY | PRE-PRODUCT CONCEPT

# ATMOS E3.S HBF-Backed Near-Data AI

Use-Case Portfolio, Industry Plays, Hyperscaler Strategy, Proof-of-Concept Plan and Product Decision Framework

Decision purpose: identify where 128 GB of persistent HBF at approximately 200 GB/s per E3.S device creates durable value, and stop use cases that can be served equally well by NVMe, CPU DRAM, conventional NPU memory, GPU partitioning or a standard accelerator.

| Field | Document position |
| --- | --- |
| Audience | Executives, architects, silicon and platform engineers, product managers, OEM teams, field teams and marketing |
| Architecture baseline | One E3.S/E3.S 2T device: one OEM NPU ASIC, up to 128 GB persistent HBF at ~200 GB/s, ~16 GB LPDDR, SRAM, PCIe Gen5/Gen6 host link |
| Scope | Enterprise, regulated industries, hyperscalers, cloud search services and neocloud platforms |
| Status | Strategic design document. All performance, power, cost and market statements are hypotheses until measured |
| Core principle | HBF must be on the steady-state data path. Cold-start reduction alone is insufficient |
| Date | September 2026 |

# Executive Summary

Recommended direction: reposition ATMOS E3.S from a generic “persistent-model inference module” to a persistent high-bandwidth neural data plane. Lead with candidate refinement, multi-vector/late-interaction retrieval, reusable document features and memory-heavy recommendation lookups. Treat dense-model streaming as conditional, and treat basic embedding, guards and model storage as supporting capabilities rather than HBF justifications.

# The central product test

ATMOS E3.S is differentiated only when a workload repeatedly reads a large active working set from HBF and reduces that data locally before crossing PCIe. If a small model is loaded once into LPDDR and then remains resident, NVMe plus LPDDR already solves the storage problem. If a model or index is never touched after warm-up, 128 GB of HBF is stranded value.

| Rank | Use case | HBF impact | Market fit | Recommendation |
| --- | --- | --- | --- | --- |
| 1 | ANN candidate refinement using full or lightly compressed vectors | Very high | Enterprise + hyperscaler search | Primary POC and product thesis |
| 2 | Late-interaction / multi-vector retrieval over selected candidates | Very high | Regulated search, cloud search, technical search | Primary POC; strongest quality story |
| 3 | Reusable document/VLM feature retrieval | High | Finance, healthcare, insurance, legal, manufacturing | Vertical solution POC |
| 4 | Recommendation and advertising embedding tables | High, if random gather is efficient | Hyperscalers, commerce, media | Hyperscaler co-design track |
| 5 | Sharded search/refinement across E3.S fleets | High aggregate | Hyperscalers, cloud/vector services, neocloud | Scale-out POC after single-device proof |
| 6 | Dense 20B–32B model weight streaming | Medium; bandwidth constrained | Private low-volume inference, batch jobs | Secondary POC, not flagship |
| 7 | Single-device MoE expert bank | Medium; technically risky | Specialist inference research | Research only |
| 8 | Embedding, guards, speech, small rerankers | Low HBF impact | Broad but undifferentiated | ModelPack/supporting feature |
| 9 | Inactive model repository / cold-start reduction | Low | Limited | Do not lead |

# What changes for each audience

| Audience | Message |
| --- | --- |
| Executives | Fund only use cases that monetize HBF through sustained bandwidth, capacity, data reduction or fleet TCO. Avoid a broad “AI accelerator” program. |
| Architects | Design around a three-stage pipeline: coarse selection, HBF-resident refinement, expensive final ranking/generation. |
| Engineers | Prove fine-grained reads, gather, dot product, max-sim, top-k, masking, decompression and queue-level tail latency before broad model enablement. |
| Product managers | Define buyer, workload, steady-state HBF bytes/query, acceptable p99 and displaced infrastructure for every SKU. |
| Marketing | Sell measured retrieval quality, bytes avoided, GPU/DRAM relief and power efficiency. Do not lead with TOPS or cold start. |
| OEM/partners | Provide E3 thermal, power, lane, BMC, telemetry and service qualification, plus a stable device runtime for partner integrations. |

# 1. Architecture Baseline and Non-Negotiable Assumptions

| Parameter | Planning assumption | Implication |
| --- | --- | --- |
| Persistent memory | Up to 128 GB HBF per E3.S device | Large read-mostly neural representations or weights can remain local |
| Sustained read bandwidth | Approximately 200 GB/s per device | Useful for selective refinement; not sufficient for repeated full 128 GB scans at interactive latency |
| Active memory | Approximately 16 GB LPDDR plus SRAM | Activations, KV cache, current tiles and temporary state must fit here |
| Compute | OEM commercial NPU ASIC, roughly 64 TOPS-class planning target | Avoids a custom Sandisk ASIC initially, but creates operator and runtime dependency |
| Host interface | PCIe Gen5/Gen6 x4 baseline; x8/2T to evaluate | Return compact results; avoid moving full representations over PCIe |
| Scale-out | Each device is an independent address space | Eight devices provide 1 TB and 1.6 TB/s only for parallel shards or independent jobs |
| Form factor | E3.S or E3.S 2T | Density must be traded against sustained accelerator cooling and power |
| Software boundary | Sandisk provides device, firmware, driver, runtime and primitives | Database, search API, orchestration and customer application remain with partners/customers |

Sanity rule: 200 GB/s is the per-device bandwidth. Eight devices can deliver an aggregate 1.6 TB/s only when eight shards execute in parallel. A single NPU or unsharded model still sees approximately 200 GB/s.

# 1.1 Where HBF sits in the memory hierarchy

ATMOS HBF should be treated as a persistent, high-capacity, high-bandwidth neural working-set tier between conventional NVMe and volatile accelerator memory. HBF is not expected to beat HBM/GDDR on bandwidth, and it will not beat NVMe on cost per bit. The product must win on the combination of persistence, local capacity, bandwidth and near-data reduction.

| Tier | Typical role | Strength | Weakness |
| --- | --- | --- | --- |
| SRAM | Immediate tiles, queues, reductions | Lowest latency and highest local bandwidth | Very small capacity |
| LPDDR | Activations, KV cache, writable workspace | Low-latency writable memory | Approximately 16 GB limits working state |
| HBF | Read-mostly vectors, features, weights, expert tables | 128 GB persistent local capacity at ~200 GB/s | Slower than HBM; endurance/update behavior matters |
| Host DRAM | Indexes, metadata, application state | Mature, flexible, coherent | Power, cost and host bandwidth |
| NVMe/object storage | Original documents, media, cold vectors, snapshots | Lowest cost and large capacity | Lower bandwidth and higher access latency |

# 1.2 Product boundaries

| In scope | Not Sandisk’s lead responsibility |
| --- | --- |
| HBF media architecture, controller, ECC, reliability and telemetry | Customer-facing RAG or search API |
| E3.S electrical/mechanical/thermal device | Replacement for Milvus or a cloud search control plane |
| Linux driver, queues, DMA, firmware lifecycle and health | Workflow orchestration such as LangGraph |
| NPU runtime and supported low-level primitives | Document parsing, chunking and application logic |
| Reference integration and partner enablement kit | A universal model-serving framework on day one |
| Measured workloads, reference designs and claims governance | Training, large-scale fine-tuning or frontier-model replacement |

# 2. Current State Without ATMOS and the Change Hypothesis

Today, enterprise and cloud retrieval stacks combine CPU/DRAM query nodes, NVMe or object storage, optional GPUs for ANN and reranking, and GPU/TPU/custom accelerators for heavy ranking or generation. Milvus separates storage, coordination and query execution, which provides a natural integration point for a specialized data-plane backend. GPU indexes can improve throughput, but Milvus documentation cautions that GPU use does not automatically improve single-query latency and is most beneficial under high request pressure or larger query batches. [S1][S2][S3]

| Dimension | Typical state today | ATMOS change hypothesis | Proof required |
| --- | --- | --- | --- |
| Index capacity | Hot index in host DRAM or GPU memory; full vectors may live on SSD | Keep richer 50–110 GB shard local and persistent | Usable capacity, recovery and update model |
| Latency | ANN may be fast; refinement/reranking adds stages and queueing | Do selective reads and scoring in one device | End-to-end p99, not media-only latency |
| Throughput | Scale by CPU cores, replicas, GPUs or distributed shards | Add independent E3.S refinement shards | Linear scaling and no PCIe/backplane bottleneck |
| Quality | Compression and small candidate counts can reduce recall | Keep richer vectors and more candidates | Recall/NDCG at fixed p99 and power |
| Cost | DRAM-heavy nodes or GPU memory reserved for retrieval | Reduce DRAM, SSD I/O and expensive ranker work | TCO at customer load and HA reserve |
| Power | CPU/GPU fleets often overprovisioned for tail latency | Use lower-power role-specific devices | Accepted queries/W and rack thermal impact |
| Recovery | Indexes/models are read from persistent storage into DRAM/VRAM | Restart with data already resident on HBF | Time to ready and operational simplicity |
| Data movement | Full vectors/features cross DDR, PCIe or network boundaries | Return only IDs, scores and compact top-k | Bytes/query and CPU/GPU interconnect relief |

No generic numerical latency or cost promise is credible before fixing dataset size, dimensions, datatype, index, recall target, candidate count, filter selectivity, concurrency, model, power and hardware. All POCs must compare at equal quality and equal SLO.

# 3. Enterprise Use-Case Portfolio

# 3.1 Use Case E1: HBF-Resident Candidate Refinement

Best-fit customer: a bank, healthcare network, insurer, legal firm, manufacturer or government organization with a moderate but high-value corpus. Milvus performs metadata filtering and coarse ANN retrieval. ATMOS holds full or lightly compressed document vectors for selected candidates, performs exact scoring and local top-k, and returns only compact IDs and scores.

Why HBF is active: every query fetches the rich representation of 1,000–10,000 candidates from HBF. The data is reused continuously, not loaded once.

| Element | Example design |
| --- | --- |
| Corpus | 5–30 million chunks, depending on representation and replication |
| Milvus role | Collection lifecycle, metadata, filters, coarse ANN, consistency, routing and result assembly |
| ATMOS HBF | Full vectors, refinement vectors, codebooks, document-side model features |
| NPU work | Gather, normalization, inner product/cosine, top-k, optional lightweight rerank |
| LPDDR/SRAM | Query vectors, candidate lists, temporary score buffers and reductions |
| Result | Top 50–200 IDs and scores returned to Milvus or the application tier |
| Business outcome | Higher recall at fixed p99; less DRAM/GPU memory; fewer weak candidates sent to the LLM |

Illustrative sizing: 5,000 candidates × 1,024 dimensions × 2 bytes is 10.24 MB per query. The media-transfer floor at 200 GB/s is about 0.051 ms, but real latency will be dominated by access pattern, queueing, scoring, top-k, host overhead and software. The POC must measure useful goodput and p99 rather than quote the arithmetic floor.

# 3.2 Use Case E2: Late-Interaction / Multi-Vector Retrieval

Best-fit customer: regulated or technical search in which terminology and clause-level distinctions matter. Instead of one vector per chunk, the system uses multiple token-level or sub-passage vectors. Milvus narrows the corpus, and ATMOS scores richer document-side representations using max-sim or related aggregation.

| Vertical | Real workflow | Why richer retrieval matters |
| --- | --- | --- |
| Financial services | Policy, regulation, audit, investigation and communication search | Similar terminology can represent very different obligations, exceptions and controls |
| Healthcare | Clinical policy, authorization criteria, coding guidance and care pathway search | Precise exclusions, conditions, age criteria and codes matter |
| Legal | Contract clause matching, discovery and precedent search | Phrase-level meaning and exceptions are easily lost in one-vector compression |
| Manufacturing | Failure reports, specifications, corrective actions and procedure search | Part numbers, conditions and failure signatures require fine-grained matching |
| Pharma/life sciences | Trial, protocol, publication and safety-signal retrieval | Entity-rich technical language benefits from token-level evidence |

Illustrative candidate volume: 5,000 candidates × 32 token vectors × 128 INT8 dimensions is 20.48 MB per query. At 200 GB/s the media-transfer floor is about 0.10 ms, again excluding irregular access and compute. This is a better fit than scanning a complete 50–100 GB corpus.

# 3.3 Use Case E3: Reusable Document and VLM Feature Store

Best-fit customer: document-heavy operational processes. Original PDFs and images remain on NVMe or object storage. ATMOS stores repeatedly queried derived features such as page embeddings, layout vectors, table representations, OCR-derived features and image-region embeddings.

| Industry | Example | Steady-state HBF access |
| --- | --- | --- |
| Insurance | Find similar claims, repair patterns, policy exceptions and fraud evidence | Read page/image/claim features for candidate cases |
| Banking/mortgage | Compare forms, detect missing evidence, match clauses and review exceptions | Read layout, field and document vectors repeatedly |
| Healthcare | Retrieve similar cases and policy evidence; search tables and scans | Read multimodal page and clinical representations |
| Semiconductor/manufacturing | Find similar defects, microscope images, reports and root causes | Read image and document features for every investigation |
| Legal/compliance | Search tables, clauses, exhibits and scanned attachments | Read rich features after textual candidate generation |

The HBF value disappears if each document is processed only once. The product must target repeated search, comparison and rescoring of reusable features.

# 3.4 Use Case E4: Dense Model Weight Streaming

Best-fit customer: private, low-concurrency or background inference where a 20B–32B quantized model must run without dedicating a large GPU. Model weights remain in HBF and are streamed layer by layer; LPDDR holds activations, KV cache, current tiles and workspace.

| INT8 model | Approx. weight size | Ideal bandwidth ceiling at 200 GB/s | Position |
| --- | --- | --- | --- |
| 8B | 8 GB | 25 tokens/s | Usually fits LPDDR; HBF value weak after warm-up |
| 13B | 13 GB | 15.4 tokens/s | May fit tightly; KV and runtime reduce feasibility |
| 20B | 20 GB | 10 tokens/s | Possible low-concurrency target |
| 32B | 32 GB | 6.25 tokens/s | Background / low-volume target |
| 70B | 70 GB | 2.9 tokens/s | Generally unattractive on one device |

These are upper bounds before attention, KV traffic, controller efficiency, graph breaks, quantization, thermal throttling and NPU utilization. Dense serving is a secondary comparison workload, not the lead product thesis.

# 3.5 Supporting, Low-HBF-Impact Workloads

| Workload | Value | Why not a lead HBF story |
| --- | --- | --- |
| Query embedding | Frees GPU and isolates p99 | Model usually fits in LPDDR and remains resident |
| Batch document embedding | Separates ingestion from interactive workloads | Data streams from host; 128 GB capacity is not central |
| Guard/classifier | Physical isolation and predictable capacity | Small model and small working set |
| Standalone cross-encoder reranker | Dedicated NPU avoids GPU contention | Weights are small; value improves only with resident features |
| Speech ASR/TTS | Potential private, low-power service | Primarily compute and operator support, not HBF capacity |
| Inactive model repository | Fast activation and persistence | NVMe solves infrequent loading more economically |

# 4. Industry Vertical Plays

| Vertical | Priority workloads | ATMOS pattern | Primary KPIs | Key qualification |
| --- | --- | --- | --- | --- |
| Financial Services | Compliance/evidence retrieval; fraud and investigation similarity; research and policy search | Candidate refinement + late interaction | NDCG/Recall, evidence precision, p99, prompt-token reduction, analyst review time | Security, tenant isolation, signed firmware, deterministic audit trace |
| Healthcare | Clinical-policy search; prior authorization evidence; coding guidance; similar-case retrieval | Late interaction + reusable document features | Recall of qualifying criteria, unsupported-answer rate, p99, clinician review time | Data residency, access filtering, validation and careful workflow boundaries |
| Insurance | Claims similarity; policy interpretation; fraud triage; document completeness | VLM/document feature store + refinement | Case retrieval precision, manual touch reduction, throughput per watt | Image/document feature support and frequent reuse |
| Legal | Clause matching; discovery; precedent retrieval; contract comparison | Multi-vector retrieval + neural reranking | NDCG/MRR, evidence precision, missed-clause rate, p99 | High-quality dataset and domain evaluation |
| Manufacturing & Semiconductor | Failure analysis; corrective-action search; spec/procedure retrieval; image similarity | Document/VLM features + late interaction | Top-k root-cause evidence, time-to-find, false match rate | Multimodal operators and domain-specific embeddings |
| Pharma & Life Sciences | Protocol, trial, publication and safety-signal retrieval | Late interaction + entity-rich features | Evidence recall, entity fidelity, review efficiency | Regulatory validation and provenance |
| Retail & E-commerce | Product similarity, catalog search, attribute matching and recommendation features | Vector refinement + embedding tables | Conversion proxy, recall, QPS/W, candidates reduced | Frequent updates and write/endurance design |
| Media & Entertainment | Video scene, image and audio retrieval; duplicate detection | Reusable multimodal feature shards | Search quality, clip/frame retrieval latency, cost per million queries | Large feature footprint and offline ingestion pipeline |
| Cybersecurity | Alert, threat, malware and incident similarity; threat-intel retrieval | Candidate refinement + multi-vector search | Detection recall, analyst time, p99, false positives | Rapid updates, tenant isolation and freshness |
| Government / Sovereign | Private records search; policy evidence; air-gapped knowledge systems | HBF retrieval + optional dense specialist model | Data-residency compliance, evidence precision, availability | Long lifecycle, secure update and offline operations |

# 4.1 Vertical ranking

| Rank | Vertical | Reason to lead |
| --- | --- | --- |
| 1 | Financial services | High value per correct result, moderate-to-large corpora, private infrastructure and strong evidence requirements |
| 2 | Legal and compliance | Retrieval quality and phrase-level matching matter more than raw scale |
| 3 | Manufacturing / semiconductor | Rich technical documents and images; strong internal design-partner potential |
| 4 | Healthcare | High need, but qualification, governance and workflow integration are demanding |
| 5 | Insurance | Multimodal claims workflow offers reusable feature-store value |
| 6 | Retail / commerce | Strong scale, but update rate and incumbent custom infrastructure raise the bar |
| 7 | Media | Large multimodal features fit HBF, but ingestion and indexing are complex |
| 8 | Cybersecurity | Promising repeated similarity workloads, but freshness and update rates must be proven |

# 5. Hyperscaler, Cloud Search and Neocloud Portfolio

At hyperscaler scale, ATMOS is not an appliance. The device becomes a fleet-level data-reduction tier. Each module owns a 128 GB shard and locally converts a large representation set into a small ranked result set before data reaches expensive GPU/TPU/custom-ASIC stages. Search systems naturally use staged retrieval and ranking because ranking the entire corpus is computationally prohibitive. [S4]

# 5.1 H1: Full-Vector Refinement After Compressed ANN

Compressed or approximate retrieval selects candidates; ATMOS stores full or lightly compressed vectors and performs exact rescoring. This directly attacks the movement of full vectors from SSD/DRAM into GPU memory. Recent HBF-augmented vector-engine research models this architecture and reports substantial latency/throughput gains, but those results are simulations and must not be used as ATMOS claims. [S5]

| Current tier | ATMOS insertion | Fleet outcome hypothesis |
| --- | --- | --- |
| CPU/DRAM or GPU ANN | Candidate IDs routed to matching HBF shards | Higher recall without expanding GPU memory |
| SSD full vectors | Full vectors persist on ATMOS HBF | Fewer SSD reads and less data movement |
| GPU exact rerank | NPU performs exact scoring and local top-k | GPU reserved for heavier rankers |
| Network result fan-in | Only local top-k returned | Lower east-west bytes per query |

# 5.2 H2: Search Ranking Funnel

ATMOS can sit between broad retrieval and expensive neural ranking. The economic unit is not “ATMOS QPS”; it is expensive candidates eliminated per watt and per dollar.

Millions/billions of items → coarse retrieval → 10,000 candidates → ATMOS refinement/late interaction → 300–1,000 candidates → GPU/TPU heavy ranker → final results

# 5.3 H3: Image, Video and Product Representation Search

Offline GPU pipelines generate embeddings, while ATMOS stores persistent image, frame, product and region representations for online refinement. This is relevant to reverse-image search, visual product search, duplicate detection, copyright matching, scene retrieval and safety candidate retrieval. Large-scale visual-search pipelines already demonstrate that optimization of preprocessing and embedding can materially change throughput and TCO, but ATMOS’s niche is the persistent online representation tier rather than media decoding. [S6]

# 5.4 H4: Recommendation and Advertising Embedding Tables

Recommendation services perform frequent sparse gathers from user, item, ad and contextual embedding tables. These tables are memory-capacity and bandwidth intensive. Processing-in-memory research reports that local aggregate bandwidth can reduce DLRM latency relative to CPU-only and CPU-GPU baselines. [S7]

| Requirement | Why critical |
| --- | --- |
| Fine-grained, highly concurrent HBF reads | Embedding access is irregular, not a sequential weight stream |
| Gather, pool and reduction operators | A dense matrix NPU alone will not accelerate the table path |
| Hot-key management and load balancing | Skew can create hot devices and tail latency |
| Fast update/refresh path | User/item embeddings change more frequently than search documents |
| Endurance and write-amplification model | HBF persistence is helpful only if updates remain economically safe |
| Local lightweight ranker | Data reduction before the primary ranking ASIC creates the value |

# 5.5 H5: Semantic Retrieval Cache

ATMOS could store read-mostly query embeddings, retrieved evidence, validated result fragments and retrieval plans. A semantic cache hit can avoid repeated retrieval, reranking and generation. This is attractive at fleet scale, but cache freshness, permission isolation, expiration, write endurance and versioning are first-class requirements.

# 5.6 H6: Query Fan-Out and Multi-Retriever Search

Agentic and generative search may produce several subqueries for one user request. Fan-out creates natural parallelism across ATMOS shard groups. The aggregate bandwidth of many independent E3.S devices becomes useful, provided routing and tail-latency control are strong.

# 5.7 H7: Neocloud Retrieval-as-a-Service Nodes

Neocloud providers can offer retrieval-optimized bare-metal or managed nodes without competing directly in frontier GPU supply. A validated server could combine CPU/DRAM control, NVMe cold storage, 8–16 ATMOS devices for persistent retrieval shards, and a smaller GPU pool for generation or heavy ranking.

| Provider type | Potential offer | Customer value |
| --- | --- | --- |
| GPU neocloud | Retrieval sidecar attached to GPU instances | More GPU memory and compute available for generation |
| Storage cloud | HBF-backed vector refinement node | Higher-value storage and search service |
| Sovereign cloud | Private RAG/search node with local persistence | Data residency and serviceability |
| Specialized AI cloud | Late-interaction retrieval service | Quality-oriented differentiation |
| Managed vector provider | ATMOS-enabled query node SKU | Persistent rich-vector capacity per node |

# 5.8 Hyperscaler ranking

| Rank | Hyperscaler use case | Impact | Risk |
| --- | --- | --- | --- |
| 1 | Full-vector candidate refinement | Very high | Requires efficient gather/top-k and data layout |
| 2 | Search ranking funnel / late interaction | Very high | Needs integration with proprietary serving stack |
| 3 | Recommendation embedding tables | Very high if supported | Random access, updates and hot-key balance |
| 4 | Multimodal representation search | High | Offline/online pipeline integration |
| 5 | Sharded semantic cache | Medium-high | Freshness, writes and permissions |
| 6 | Query fan-out shard pool | Medium-high | Network fan-out and tail latency |
| 7 | Dense specialist-model hosting | Medium | Weak HBF utilization for small models |
| 8 | MoE expert bank | Research | PCIe and token-level synchronization |

# 6. Scale-Out Architecture

Correct aggregate interpretation: N devices provide N × 128 GB capacity and N × 200 GB/s aggregate bandwidth only when data and work are partitioned. Each individual shard remains capped by one device’s bandwidth and compute.

| Topology | Best use | Advantages | Risks |
| --- | --- | --- | --- |
| Independent replicated devices | Embedding/rerank service replicas | Simple scheduling and fault isolation | Low HBF utilization for small models |
| Corpus-sharded devices | Vector/multi-vector refinement | Linear capacity and aggregate bandwidth | Fan-out, global top-k and slowest-shard tail |
| Role-specific devices | Retrieval, rerank, VLM, dense model per device | Clear workload ownership | Fleet fragmentation |
| Homogeneous device pool | Hyperscaler or neocloud service | Operational simplicity and load balancing | Requires one dominant, repeatable workload |
| Cross-device model sharding | Large dense/MoE model | Aggregate capacity | PCIe latency, collectives and runtime complexity |

# 6.1 Reference sharded retrieval path

Query router maps filters and index partitions to a subset of ATMOS devices.

Each device reads only local candidate representations and generates a local top-k.

Host or search service merges local top-k lists into a global result set.

Optional heavy ranker processes the reduced candidate set.

Replication handles device maintenance and availability without requiring shared HBF semantics.

# 6.2 Scale-out engineering requirements

| Area | Required work |
| --- | --- |
| Routing | Shard map, locality-aware dispatch, filter-aware pruning and request hedging |
| Top-k merge | Deterministic local/global score normalization and efficient merge |
| Availability | Replica placement, rebuild, failover and degraded-mode behavior |
| Updates | Segment versioning, immutable snapshots, delta path and background refresh |
| Backplane | PCIe switch map, uplink oversubscription and host ingress/egress budget |
| Tail latency | Deadline propagation, straggler handling and hot-shard mitigation |
| Observability | Bytes/query, useful HBF bandwidth, queue depth, throttling and result quality |
| Security | Tenant isolation, encryption, secure erase and attested firmware |

# 7. Proof-of-Concept Portfolio and Exit Criteria

POC discipline: every POC must compare equal-quality outputs at a fixed service-level objective. A demo that only proves a model or index fits is not sufficient.

| ID | POC | Scope | Exit criterion |
| --- | --- | --- | --- |
| POC 0 | Memory-path characterization | Measure sequential/random read bandwidth, granularity, concurrency, energy, endurance and HBF-to-NPU goodput | Sustained useful bandwidth under representative gather patterns; no hidden LPDDR/PCIe bottleneck |
| POC 1 | Candidate refinement | Milvus coarse ANN returns 1K–10K IDs; ATMOS fetches full vectors, exact-scores and top-k | Better recall/NDCG at equal p99, or lower power/cost at equal quality versus CPU/GPU baseline |
| POC 2 | Late-interaction retrieval | HBF stores multi-vector document features; NPU executes max-sim and aggregation | Material quality gain within target p99 and power; HBF read continuously |
| POC 3 | Document/VLM feature search | Store reusable page/layout/image features and repeatedly search/rescore | Reduced host/GPU data movement and improved vertical outcome |
| POC 4 | Dense-model stream | 20B–32B quantized model remains in HBF and streams each token | Measured TPOT, energy/token and useful HBF bandwidth beat NVMe offload and meet target workload |
| POC 5 | Eight-device sharded refinement | Partition corpus across devices and merge local top-k | Near-linear throughput, bounded tail latency and no backplane saturation |
| POC 6 | Recommendation gather | HBF holds hot embedding tables; execute gather/pool/light rank | Competitive p99 and QPS/W under skew and update load |
| POC 7 | Operational qualification | Reset, firmware update, thermal soak, hot/planned service and data recovery | Production-grade telemetry, recovery and no sustained throttling |

# 7.1 POC 1 detailed test matrix

| Variable | Test points |
| --- | --- |
| Dataset | 10M, 25M and capacity-bound shard |
| Vector layout | INT8, FP16; 768 and 1,024 dimensions |
| Candidate count | 1K, 5K, 10K and 50K |
| Access pattern | Contiguous segment, batched random gather, filter-pruned gather |
| Quality | Recall@10/100, NDCG@10, MRR, context precision |
| Latency | p50/p95/p99 including host dispatch and merge |
| Throughput | QPS at concurrency 1, 8, 32, 128 and saturation |
| Efficiency | Queries/W, useful GB/s, CPU usage and PCIe bytes/query |
| Controls | Milvus CPU/DRAM; NVMe-backed; GPU index/refinement; optional CXL memory tier |
| Operational | Cold boot, warm restart, shard replacement and rebuild |

# 7.2 POC 2 detailed correctness and quality

Use an industry-domain evaluation set with judged query-document relevance, not only generic ANN datasets.

Compare one-vector dense retrieval, compressed ANN plus exact refinement, and multi-vector late interaction.

Hold the final LLM and prompt constant to isolate retrieval impact.

Measure context precision, grounded-answer rate and prompt-token reduction in addition to retrieval metrics.

Report the incremental latency and power cost of each quality stage.

Document unsupported operators, CPU fallbacks and graph breaks.

# 8. Work Required from Sandisk

# 8.1 Hardware and HBF architecture

| Work item | Deliverable | Gate |
| --- | --- | --- |
| HBF characterization | Bandwidth/latency distributions by access size, queue depth and random/sequential pattern | Representative useful goodput reaches workload target |
| Memory controller | Gather support, high outstanding requests, ECC, bad-block management and QoS | No bottleneck on candidate-access trace |
| Endurance model | Read/write amplification, update policy and retention | Search index refresh is economically safe |
| LPDDR/SRAM budget | Capacity, bandwidth and contention analysis | Activations, buffers and queues fit with reserve |
| Thermal/power vehicle | E3.S and 2T airflow, sustained read + NPU load, ambient corners | No sustained throttling in qualified server |
| PCIe design | x4/x8 trade, DMA, ATS/PASID, SR-IOV/P2P options | Host overhead and ingress/egress within SLO |
| RAS/security | Secure boot, signed firmware, telemetry, reset, sanitization | Enterprise/hyperscaler baseline met |

# 8.2 OEM NPU selection and contract requirements

| Capability | Minimum requirement |
| --- | --- |
| Dense math | INT8/FP16/BF16 dot products, normalization and matrix/vector operations |
| Search primitives | Gather, masked scoring, top-k, max-sim, reduction and optional decompression |
| Execution | Multiple queues, bounded completion latency and concurrent DMA/compute |
| Memory | Direct HBF access or efficient staging without host round trips |
| Compiler | Stable custom-op path and transparent unsupported-op reporting |
| Runtime | C/C++ device library and versioned ABI; Sandisk need not create customer APIs |
| Roadmap/control | Long-term supply, bug-fix SLA, security response and access to performance counters |

# 8.3 Firmware, driver and runtime

Linux enumeration, IOMMU-safe DMA, memory registration and command/completion queues.

HBF object/segment registration, immutable snapshot activation and atomic version switch.

Queue priorities, admission, timeout, cancellation, backpressure and recovery.

Device reset, watchdog, fault isolation, firmware update/rollback and health telemetry.

Counter set for HBF useful bytes, read amplification, queue stalls, NPU utilization, LPDDR traffic, PCIe traffic, errors and thermal throttling.

A stable low-level SDK for integration partners, avoiding ownership of a customer-facing search or RAG API.

# 8.4 Software and ecosystem integration

| Partner surface | Sandisk provides | Partner provides |
| --- | --- | --- |
| Milvus / vector database | Device backend SDK, sample Knowhere-style adapter, reference kernels, qualification | Database semantics, control plane, routing, metadata and customer API |
| Model/runtime partner | Compiler hooks, memory planner constraints and device library | Model import, packaging and serving framework integration |
| OEM/server | Electrical, thermal, firmware and management specs | Chassis, BIOS/BMC, lane/cooling and service qualification |
| Hyperscaler | Device traces, counters, low-level primitives and co-design support | Proprietary search/ranking integration and fleet orchestration |
| Neocloud | Validated server SKU, driver, telemetry and support package | Provisioning, billing, managed service and customer support |

# 8.5 Product management and marketing

Define a workload-specific SKU thesis, not a generic AI-accelerator thesis.

Create an evidence ledger separating architecture targets, modeled results, prototype measurements and production claims.

Price against displaced infrastructure and operational outcome, not only capacity or TOPS.

Publish fit/no-fit guidance, including the 200 GB/s dense-streaming ceiling and the difference between per-device and fleet aggregate bandwidth.

Build vertical reference stories around evidence quality, GPU/DRAM relief, bytes avoided and power, not cold start.

Recruit one enterprise, one hyperscaler/search provider, one vector/database partner and one OEM platform as design partners.

# 9. Recommended Program Roadmap

| Phase | Timing | Objectives | Decision output |
| --- | --- | --- | --- |
| 0. Reframe | Weeks 1–2 | Freeze HBF-centric thesis, use-case ranking and no-fit list | Approved scope and success metrics |
| 1. Characterize | Weeks 1–6 | Memory path, NPU primitive matrix, thermal model and workload traces | Feasibility scorecard |
| 2. Single-device proof | Weeks 5–12 | Candidate refinement and late-interaction emulation/prototype | Measured reason to use 128 GB HBF |
| 3. Vertical proof | Months 4–6 | Finance/manufacturing/healthcare dataset and outcome tests | Design-partner evidence |
| 4. Scale-out proof | Months 6–9 | 8-device shard routing, top-k merge, HA and backplane study | Reference server architecture |
| 5. Productization | Months 9–18 | Silicon/device, firmware, driver, OEM and ecosystem qualification | Launch decision and claims package |

# 9.1 Decision gates

| Gate | Advance | Hold/redesign | Stop |
| --- | --- | --- | --- |
| G1: HBF relevance | Workload uses >32 GB active representation and reads HBF continuously | Only some stages use HBF | Value is cold start or inactive storage |
| G2: Useful bandwidth | Representative access pattern achieves required goodput | Controller/layout changes needed | Random/gather behavior destroys performance |
| G3: Quality/SLO | Beats baseline at equal quality/SLO or improves quality within SLO | Advantage only under narrow conditions | CPU/GPU remains clearly superior |
| G4: Power/thermal | Sustained workload fits qualified E3 profile | 2T or lower clock required | No feasible E3 deployment |
| G5: Economics | Displaces measurable DRAM/GPU/SSD I/O or ranker work | Pricing sensitivity wide | No credible TCO advantage |
| G6: Ecosystem | Partner integration path and OEM platform committed | One material blocker | Sandisk would need to own full application stack |

# 10. Role-Based Guidance

| Role | What to remember | Immediate action |
| --- | --- | --- |
| Executive | ATMOS wins only where HBF is active after warm-up | Fund two HBF-first POCs and enforce stop criteria |
| Chief architect | Design staged, near-data reduction rather than a small GPU | Freeze refinement and late-interaction architecture |
| Hardware engineer | 200 GB/s media bandwidth is not useful goodput | Characterize gather, concurrency, power and thermal corners |
| Firmware engineer | Tail latency and recovery are product features | Define queues, counters, resets and atomic data activation |
| AI/compiler engineer | Search primitives are as important as Transformer operators | Close gather/top-k/max-sim/custom-op gaps |
| Database/search engineer | Milvus remains control plane; ATMOS is a backend | Prototype candidate-ID to local-feature scoring path |
| Product manager | Every use case needs buyer, SLO and displaced cost | Build workload scorecards and no-fit map |
| Marketing | Do not sell raw TOPS or aggregate bandwidth | Lead with rich retrieval, data reduction and measured TCO |
| Field/OEM | Existing E3 bay is not automatically accelerator-ready | Qualify power, airflow, lanes, BMC and service model |

# 11. Key Risks and Mitigations

| Risk | Why it matters | Mitigation |
| --- | --- | --- |
| OEM ASIC lacks search primitives | Best HBF use cases become inaccessible | Make gather/top-k/max-sim/custom ops selection gates |
| 200 GB/s only achieved sequentially | Candidate refinement is random or batched gather | Trace-driven characterization and data-layout co-design |
| HBF update/endurance limits | Indexes and embeddings change | Immutable segments, delta tier, background rebuild and endurance modeling |
| E3 thermal envelope is insufficient | Sustained NPU + HBF differs from SSD duty cycle | Evaluate 2T, clocks, airflow and workload power modes |
| GPU baseline is already better | Mature ecosystem and high bandwidth | Compete on persistent capacity, data reduction, power and role specialization |
| Software integration burden shifts to Sandisk | Company does not want to own APIs | Stable backend SDK plus funded partner strategy |
| Moderate enterprise volume is too low | CPU/DRAM remains cheaper | Target high-value quality workloads and avoid horizontal claims |
| Hyperscaler has custom silicon | High replacement barrier | Offer HBF/media advantage and co-design as a memory/data-plane component |
| Cross-device model story expands scope | PCIe lacks coherent high-speed model fabric | Keep E3.S to shards; reserve integrated card for tightly coupled models |
| Marketing overstates 1.6 TB/s | Aggregate figure misunderstood | Always state 200 GB/s per device and conditions for aggregate scaling |

# 12. Final Recommendation

Proceed with an HBF-first feasibility program. Make candidate refinement and late-interaction retrieval the two primary product POCs. Add a vertical reusable-feature POC and a hyperscaler recommendation/search co-design track. Keep dense-model streaming as a measured secondary capability. Stop any use case whose value reduces to cold start, inactive model storage or running a small model that fits entirely in LPDDR.

| Decision | Recommendation |
| --- | --- |
| Primary product identity | Persistent high-bandwidth neural retrieval and model-memory tier |
| Primary enterprise use | Milvus-integrated candidate refinement and high-accuracy multi-vector retrieval |
| Primary verticals | Financial services, legal/compliance, manufacturing/semiconductor, healthcare and insurance |
| Primary hyperscaler use | Full-vector refinement, ranking-funnel reduction and memory-heavy embedding-table access |
| Primary neocloud offer | Retrieval-optimized server SKU beside GPU generation nodes |
| Secondary capability | 20B–32B dense model streaming for low-volume/private inference |
| Research only | Single-device MoE expert bank and semantic cache |
| Reject as headline | Embedding alone, guard models, cold start, inactive model archive and cross-device MoE |

Management decision requested: authorize a gated 12-week program whose success criterion is at least one measured workload that continuously consumes HBF and demonstrates a material quality, SLO, power or TCO advantage over CPU/DRAM, NVMe and GPU alternatives.

# Appendix A. Benchmark and TCO Scorecard

| Category | Required metrics |
| --- | --- |
| Quality | Recall@10/100, NDCG@10, MRR, context precision, grounded-answer rate and task-specific accuracy |
| Latency | p50/p95/p99, queueing, device time, host time, fan-out and merge |
| Throughput | QPS, candidates/s, pairs/s, tokens/s and saturation curve |
| Memory | HBF occupancy, useful bytes/read, read amplification, LPDDR occupancy and cache hit |
| Compute | NPU utilization, operator mix, graph breaks and CPU fallback |
| Interconnect | PCIe ingress/egress, switch uplink usage and network bytes/query |
| Power | Idle, active, peak, queries/W and energy per accepted result |
| Operations | Load/recovery time, update throughput, rebuild, reset and firmware rollback |
| Economics | Device/server cost, rack density, HA reserve, software/support and cost per successful query |
| Comparators | CPU/DRAM, NVMe, GPU, GPU-partitioned service and optional CXL memory tier |

# Appendix B. Source Notes

The following public sources support market and architectural context. Research results describe the cited systems and are not ATMOS product claims.

| Source | Relevance | Location |
| --- | --- | --- |
| [S1] Milvus Architecture Overview | Milvus documents a disaggregated architecture with proxy, coordinator, worker/query nodes and persistent storage. | https://milvus.io/docs/architecture_overview.md |
| [S2] Milvus Storage/Computing Disaggregation | Milvus describes query nodes loading historical data and data nodes handling compaction/index building. | https://milvus.io/docs/four_layers.md |
| [S3] Milvus GPU Index Documentation | Milvus notes GPU indexes can improve throughput/high recall, but may not reduce latency without sufficient query pressure or batching. | https://milvus.io/docs/gpu_index.md |
| [S4] Google Cloud Retrieval and Ranking Overview | Describes staged retrieval and ranking, with retrieval narrowing the corpus before ranking. | https://docs.cloud.google.com/generative-ai-app-builder/docs/ranking-overview |
| [S5] HAVEN: High-Bandwidth Flash Augmented Vector Engine | 2026 research proposal for HBF-augmented GPU/vector refinement; reports modeled gains for billion-scale datasets. | https://arxiv.org/abs/2603.01175 |
| [S6] NVIDIA / Microsoft Bing Visual Search Optimization | Reports 5.13x end-to-end throughput improvement in a billion-image visual embedding pipeline after GPU software optimization. | https://developer.nvidia.com/blog/optimizing-microsoft-bing-visual-search-with-nvidia-accelerated-libraries/ |
| [S7] UpDLRM | Research on processing-in-memory acceleration for recommendation embedding lookups and irregular memory access. | https://arxiv.org/html/2406.13941v1 |
| [S8] DistVS | 2026 compute-memory-disaggregated vector-search research with tiered vector precision and reported throughput improvement. | https://www.usenix.org/conference/nsdi26/presentation/yin |
| [S9] Intelligent Knowledge Store | Near-memory CXL retrieval research reporting exact nearest-neighbor and end-to-end RAG improvements. | https://arg.csl.cornell.edu/papers/iks-asplos2025.pdf |
| [S10] Fusion-ANNS | CPU/GPU/SSD collaborative filtering and reranking research reporting QPS and cost-efficiency improvements. | https://www.usenix.org/conference/fast25/presentation/tian-bing |
| [S11] ATMOS E3.S Product and Workstream Proposal | Internal source for architecture assumptions, product boundaries, readiness items and planning targets. | Internal document supplied for this analysis |