# ATMOS POC Platform & Use Cases

## Overview

Build a reusable evaluation platform that compares RTX + NVMe/GDS, RTX + CXL DRAM, and ATMOS + working memory + HBF. Evaluate both the workload result and the practical requirements to install, connect, cool, manage, and support the hardware. POC goal is to verify that how do the three architectures compare in sustainable capacity, latency, memory pressure, and cost as Context, Load and Document size increase with different real life use cases in Financial, Healthcare and Telecom.

| Architecture | Investigation | What it targets | Proposed test and output |
| --- | --- | --- | --- |
| RTX + accelerator memory + NVMe/GDS | Baseline E3.S SSDs | A credible enterprise storage tier | Compare supported drive counts and placement; verify actual lanes and GDS. Output: qualified SSD-to-GPU configuration. |
| RTX + accelerator memory + CXL DRAM | CXL device, if selected | Real warm DRAM control B | Confirm that the host/bay supports that CXL device, not only NVMe drives. Output: measured memory-placement and staging path. |
| ATMOS + DDR + HBF | Future HBF-only or ATMOS module | Product packaging and OEM adoption | Obtain engineering form-factor, power, thermal, protocol, and management requirements. Output: fit, engineering changes required, or incompatible. |

## Use Case Setup: -

The POC setup will be generic and should be able to handle any practical involves high concurrency LLM serving with variety of context sizes and large documents for RAG

Priority 1 — LLM serving: Dense-model controls, MoE serving, expert placement, KV reuse, and communication. No live vector database is required.

Priority 2 — RAG and retrieval: Add vector search and end-to-end retrieval only after the LLM baseline is understood. Business use cases are workload packs at the end of this document.

Approach: Start economically in cloud or an existing lab. Qualify a reconfigurable OEM server early. Repeat selected tests on physical ATMOS/HBF when available, then expand to multiple servers.

Outcome: A workload-fit map, an OEM compatibility assessment, measured data paths, and a specific list of hardware/software changes needed for deployment.

Load generator -> API service -> job queue -> fixed workflow executor
                             -> RAG service -> model-serving endpoint
                             -> tier adapter -> result store + telemetry

| Layer | Initial POC choice | Notes |
| --- | --- | --- |
| API service | FastAPI or equivalent Python service | Accepts package submissions, exposes status/results, records request metadata |
| Job queue | Redis Queue/Celery for POC; Kafka only if already available | Controls arrival rate, retries, backlog, and saturation behavior |
| Workflow/agent layer | LangGraph-style fixed workflow graph | Agentic shape, but no open-ended planning; same graph and tool responses across architectures |
| Synthetic data | Python generator for applications, borrower data, statements, tax forms, collateral records, and policies | Produces package manifests, expected fields, citations, and arrival traces |
| Document store | Local filesystem, S3-compatible object store, or MinIO | Stores synthetic source documents and immutable test corpora |
| Metadata store | PostgreSQL or SQLite for POC scale | Tracks packages, stages, timestamps, correctness checks, and result IDs |
| RAG indexing | Chunked policy and document corpus with fixed embedding model | Embeddings are precomputed; embedding quality is not part of the POC decision |
| Vector store | Qdrant preferred; FAISS acceptable for local-only runs | Same retrieved chunks must be supplied across architectures for comparison runs |
| Retrieval policy | Top-k retrieval with fixed filters and prompt template | Retrieval latency is measured, but the POC does not optimize RAG quality |
| Model serving | vLLM or SGLang for RTX controls; trace-compatible serving model for ATMOS emulator | Same model, tokenizer, prompts, output limits, and decoding settings |
| Tier adapter | Common interface for expert, KV, and document/context reads | Routes to NVMe/GDS, CXL DRAM, or ATMOS/HBF emulator without changing workflow code |
| Telemetry | OpenTelemetry plus Prometheus/Grafana or equivalent | Captures queue time, stage time, token latency, bytes moved, cache behavior, and errors |
| Load test | Locust, k6, or a Python asyncio generator | Drives low, target, burst, and saturation workloads from the frozen arrival trace |

## Expected outputs:-

| Workstream | What we will establish | Main output |
| --- | --- | --- |
| LLM/MoE serving — priority 1 | Which TP/EP layouts, expert policies, and KV configurations meet the service target | Serving topology and performance comparison |
| RAG/retrieval — priority 2 | Whether retrieval creates a separate storage opportunity or hides the inference benefit | Retrieval and end-to-end results, reported separately |
| Simulation and calibration | Which HBF/ATMOS capabilities are required and how trustworthy the predictions are | Versioned models, error report, and fit/no-fit ranges |
| Kubernetes and multi-server operation | Whether workloads can be deployed, isolated, recovered, and scaled | Deployment runbook and recovery evidence |
| OEM platform and replaceability | Which chassis, slots, bays, power, cooling, and firmware support the intended configurations | Qualified configuration and replacement matrix |
| Hardware and SoC data paths | Where bytes move and where capacity, bandwidth, queues, or controllers limit progress | Measured path map and ATMOS instrumentation requirements |

# Cloud first, OEM proof next, scale afterward

Cloud is a useful development and measurement environment. It cannot establish the mechanical, thermal, firmware, or support fit of an unreleased card inside a particular OEM server.

| Stage | Environment | What it establishes | What it does not establish |
| --- | --- | --- | --- |
| Discover | CPU simulator host and one available GPU server | Reproducible harness, workload contract, initial path measurements | ATMOS hardware performance or OEM compatibility |
| Qualify one OEM node | Exact chassis, backplane, risers, cards, drives, NICs, and BIOS | Real topology, thermal behavior, supported replacement procedure, and container paths | Cross-server scaling |
| Validate two nodes | Two compatible servers and a documented network | Actual communication, shared-cache/P–D tests, failure and recovery | Measured rack-scale performance |
| Expand selectively | Four/eight physical nodes for decision-critical cells; larger simulated topologies | Scaling limits and deployment economics | Measured behavior at untested scale |
| Refresh generations | New qualified card, memory, storage, or host revision | Regression results and updated compatibility | Automatic support for every future component |

## Minimum starting lab

Use a Linux x86_64 machine with approximately 16 vCPU and 64 GiB RAM for TokenSim development, consistent with POC 01. Borrow or rent GPU capacity for measurement. Start with one or two accelerators for functional work; use a four/eight-accelerator configuration only for the selected parallelism tests. A real CXL host/device may be a separate evaluation fixture. [S2]

Select an OEM server with a documented expansion route, but confirm which partial GPU populations are supported. Do not assume a configuration marketed for eight GPUs can be operated in every smaller population.

## Cloud preflight

Capture the exact accelerator, tenancy, host-memory allocation, storage type, filesystem, PCIe/NUMA visibility, NIC topology, driver versions, and actual direct-transfer behavior. Network block storage must not be mislabeled local NVMe. A cloud result with hidden topology is a useful bounded control, not a complete OEM qualification.

Physical delivery is conditional: If ATMOS or HBF hardware is unavailable, finish the conventional controls, simulation requirements, OEM gap list, and software tests. Do not describe the POC as a physical ATMOS deployment. If CXL hardware is unavailable, mark B as unmeasured; do not substitute a simulated B without changing its evidence label.

# OEM selection

The preferred outcome is one reusable base server for supported A/B/C configurations. The fallback is a small family of matched, qualified fixtures when CXL, power, or accelerator mechanics prevent a single-host design.

## Initial OEM investigation

| Candidate | Public basis for investigating it | What still requires confirmation |
| --- | --- | --- |
| Dell PowerEdge XE7740 | Published configuration offers eight PCIe Gen5 x16 accelerator positions and up to eight front E3.S Gen5 NVMe drives. Dell documents RTX PRO 6000 configurations. [R1, R2] | Proposed ATMOS acceptance, usable lane topology, partial population, third-party drive firmware, CXL, and supported lab modifications |
| Lenovo SR675 V3 / SR675i V3 | Published SR675i inference configuration combines eight RTX PRO 6000 GPUs and E3.S NVMe. Its major CTO components are fixed. [R5] | Whether a different configurable variant or engineering-evaluation agreement meets the replacement objective; no arbitrary substitutions assumed |
| Another OEM/ODM engineering platform | Candidate only after receiving an exact configuration and topology | Same acceptance checklist; do not select from a product-family name alone |

This is a shortlist for engagement, not a purchase recommendation or a statement that any OEM supports ATMOS today.

## Required OEM evidence before selecting a host

| Area | Evidence to obtain |
| --- | --- |
| Mechanical and power | Card dimensions, slot spacing, retention, auxiliary power, rail/current limits, PSU redundancy, and supported population |
| Connectivity | Per-slot generation/width, switches, retimers, shared upstream links, NVMe/NIC affinity, and documented CXL support |
| Thermal design | Airflow/pressure needs, fan policy, inlet limits, power caps, throttling behavior, and cooling service procedure |
| Firmware and management | BIOS/BMC versions, MMIO/BAR requirements, secure boot, inventory, sensors, firmware-update and reset mechanisms |
| Software/support | OS/driver matrix, device qualification process, engineering access, support ownership, and restrictions on substitutions |

Selection rule: Reject a candidate for this fixture if the proposed modifications cannot be documented and supported. A very fast fixed appliance may be a good reference control but a poor interchangeable-component lab.

# Hardware Swapping.

“Swappable” means a documented reconfiguration with a new configuration ID. It does not mean live removal, identical drivers, or universal electrical compatibility.

| Component | Realistic replacement boundary | Required requalification |
| --- | --- | --- |
| RTX ↔ ATMOS add-in card | Possible only if card mechanics, power, cooling, host interface, firmware, and software are compatible | Boot, enumeration, memory mapping, DMA, correctness, thermal soak, and recovery; normally power down |
| Current ↔ future RTX generation | New approved card/configuration; not an assumed drop-in | Driver/runtime, negotiated link, power/cooling, slot occupancy, model support, and baseline rerun |
| E3.S NVMe devices | Replace qualified drives in compatible bays; vary count and layout deliberately | Firmware, lane wiring, controller/RAID path, GDS, endurance, and steady-state tails |
| Host DDR DIMMs | Qualified capacity/speed/population within the CPU and motherboard limits | Channel balance, actual speed, NUMA behavior, bandwidth, and power |
| CXL memory | Qualified host, device, connector, BIOS, OS, and memory configuration | Placement, capacity, latency, memory errors, and host-to-GPU path |
| On-card LPDDR, HBM/GDDR, HBF | Treat as part of the card or board revision unless engineering explicitly provides replaceable modules | New card/profile; do not plan a host-DIMM-style memory swap |
| NoC, HIM, NPU/controller logic | Silicon/firmware revision or simulated design variant | New profile and validation; not a server field replacement |
| PCIe generation / DDR generation | End-to-end host/board capability; a slot adapter or BIOS setting cannot create missing hardware support | CPU, motherboard, riser/backplane, switches/retimers, device, and signal/power qualification |
| NIC/fabric | Qualified NIC, port, cable/optic, switch, and software combination | Link rate, RDMA, affinity, collectives, isolation, and congestion |

# Hardware experiments: connect the studies to measurements

Begin with read-only discovery. Performance, power, and fault tests run only on approved lab resources and disposable datasets. Do not disable isolation or change firmware settings simply to obtain a faster path.

| Test | What to compare | What it tells us |
| --- | --- | --- |
| H01 — Inventory and topology | Planned BOM versus actual slots, links, NUMA nodes, firmware, and device visibility | Whether the system is the configuration we think we are testing |
| H02 — Local execution memory | Bandwidth and latency for workload-relevant kernels and working-set sizes | Whether compute is waiting on its own memory rather than external storage |
| H03 — Host/device movement | Pinned host memory, NUMA-local/remote placement, asynchronous copies, and verified peer paths | Cost and availability of each staging route |
| H04 — NVMe/GDS | Engine-native cache/direct I/O; one versus multiple drives; verified GDS versus fallback | Effective delivery of missing weight/KV data and scaling limits |
| H05 — CXL | Host DRAM versus real CXL memory, placement and GPU staging | Whether capacity benefits survive the extra path |
| H06 — ATMOS internal paths | HBF → named destination; concurrent working-memory and NPU traffic | Which controller, fabric, or buffer limits the integrated design |
| H07 — Shared-path interference | Expert fetch + KV restore + GPU communication + background storage, alone and together | Whether individually fast components contend in the real service |
| H08 — Physical operation | Sustained load, cold/warm restart, permitted resets, power caps, and full population | Thermal stability, recovery, and serviceable configurations |

# Priority 1: LLM-only workload suite

Start with prepared prompts and deterministic traffic. Documents may supply prompt text, but no live retrieval, embedding service, external tool, or vector database is needed for this track.

| Workload cell | Purpose | What we vary |
| --- | --- | --- |
| L01 — Small resident control | Verify serving correctness and establish an execution-only reference | Input/output length, batching, and arrival rate |
| L02 — Resident MoE | Compare parallelism without forcing storage misses | TP/EP layout, expert distribution, and batch shape |
| L03 — Tiered MoE | Test whether a larger lower tier helps | Fast expert-cache budget, actual misses, prefetch, and lower-tier path |
| L04 — Long context | Separate KV capacity and attention cost from expert-weight cost | Context, active sequences, KV precision/layout, and memory allocation |
| L05 — Reusable prefix / paused session | Compare retain, restore, and recompute | Reuse, pause intervals, burst resumes, and version compatibility |
| L06 — Model/adapter switch | Measure readiness and recurring switch cost | Warm/cold start, number of model versions, and reuse |
| L07 — Mixed arrivals and contention | Find sustainable load and noisy-neighbor effects | Short/long request mix, bursts, priorities, and interference |

## Model and workload selection

Use a small supported model for functional work, a supported small MoE for topology experiments, and one large MoE as the primary capacity case. The original Qwen3-235B-A22B candidate is retained as a candidate, not a promise of ATMOS support. A trillion-parameter profile is initially analytical/simulated unless hardware, weights, license, and runtime support are confirmed. New models are versioned workload entries, not unchecked names added to the title. [S1]

Start with 8K, 16K, and 32K contexts; add 64K/128K only with validated model/runtime support. Freeze the BF16/FP16 reference; evaluate supported FP8 or INT4 as separate quality-checked cells. Use short/long outputs, low/target/saturation load, and real routing traces where available. Synthetic expert distributions are sensitivity tests, not measured model behavior.

## Memory accounting is mandatory

Publish bytes used by shared weights, expert weights and replicas, active/shared KV, activations, workspaces, communication buffers, and I/O rings. Track total footprint, actually accessed working set, and bytes fetched separately. KV shape depends on the model; do not reuse a generic formula for MLA, hybrid, or sliding-window models without validation.

Primary result: Sustainable requests/second meeting the fixed correctness and latency requirements. Report output tokens/second and input-token processing separately; do not combine differently priced or differently costly work into one ambiguous “tokens/s” total.

# Genuine expert TP/EP hybrids and multi-node layouts

A model that fits is only a starting point. Test whether splitting tensors, distributing experts, or running independent replicas is the better use of the available cards.

## Define the groups before launching

TP divides tensor computation. EP places experts across ranks. PP places layer groups in successive stages. Independent replicas each contain a complete logical model, potentially spanning several cards. In some MoE runtimes, “DP” refers to attention workers that still share an expert group; it is not a count of independent complete replicas. [R9, R10]

## Proposed eight-GPU vLLM starting cells

These are candidate configurations, not executable recipes or guaranteed model fits. PP is 1. Check model divisibility, memory, kernels, transport, and the pinned runtime before running. The same logical tests apply at smaller scale when valid.

| ID | Attention TP / DP | Expert layout | Independent complete groups | Question |
| --- | --- | --- | --- | --- |
| P01 | TP8 / DP1 | EP disabled; tensor-sharded experts | 1 | Conventional full tensor sharding |
| P02 | TP4 / DP1 per deployment | EP disabled | 2 separate deployments | Does smaller TP plus replication improve service capacity? |
| P03 | TP2 / DP1 per deployment | EP disabled | 4 separate deployments | Lower communication versus duplicated weights |
| P04 | TP1 / DP8 | EP8 | 1 shared expert domain | Replicated attention plus distributed experts |
| P05 | TP2 / DP4 | EP8 | 1 shared expert domain | Sharded attention with shared expert execution |
| P06 | TP4 / DP2 | EP8 | 1 shared expert domain | More attention TP, fewer attention-DP workers |
| P07 | TP8 / DP1 | EP8 | 1 shared expert domain | Attention TP across all cards with experts distributed |

The referenced vLLM EP guide defines EP size as TP × DP in this arrangement. That does not mean multiplying by EP again to count physical GPUs, and it does not mean each expert is tensor-sharded across TP ranks. Save the actual rank groups and layer behavior from the selected release. [R9, R10]

Reject invalid cells visibly. A model too large for P03 is a capacity result, not a failed benchmark to hide. First compare resident policies, then layer in tiering so that communication changes are not mistaken for HBF gains.

The previous vLLM matrix varies attention TP and shared EP. A different question is whether to split each expert's matrices across multiple ranks while also distributing different experts across groups.

## True expert-tensor/expert-parallel combinations

For an illustrative eight-rank expert execution group, test expert-TP × expert-EP = 8, where a supported runtime exposes both dimensions. These factors describe expert placement, not an additional eightfold resource multiplier.

| Expert TP | Expert EP | Meaning | What to measure |
| --- | --- | --- | --- |
| 8 | 1 | Every expert is tensor-split over eight ranks | Collective cost and per-rank footprint |
| 4 | 2 | Two expert-owner groups, four ranks per expert | Balance of dispatch and within-expert synchronization |
| 2 | 4 | Four expert-owner groups, two ranks per expert | Local group placement and communication overlap |
| 1 | 8 | Full individual experts distributed across ranks | Dispatch/combine tails and expert-load imbalance |

TensorRT-LLM documents an expert TP/EP hybrid concept, but the available controls differ by backend, model, and release. Do not transfer its flags or semantics to vLLM or ATMOS without validation. ATMOS rows remain modeled or unsupported until its runtime provides the required operations. [R11]

## Selected follow-on topologies

Two nodes: Compare independent node-local serving groups against one group spanning both nodes. Keep frequent TP traffic inside a fast local domain where feasible, but test rather than assume the best mapping.

PP alternative: Compare layer-stage handoff across nodes with wide TP/EP. Record pipeline idle time, microbatch behavior, memory fit, and synchronization. PP/EP combinations require their own runtime support check.

Prefill/decode separation: Compare colocated execution with separate homogeneous worker pools. Measure KV-transfer size, latency, conversion cost, and network contention. Mixed RTX-prefill/ATMOS-decode is a later feasibility study: matching API shape does not guarantee matching KV representation or model numerics. [R16]

Rack domain: Model the actual scale-up boundary and cross-rack links. Do not assign NVLink-class bandwidth to an RTX PCIe server. Validate two real nodes before promoting larger-scale predictions.

ATMOS replication hypothesis: A full model in HBF may make smaller serving groups possible. Demonstrate working-memory fit, operator support, compute capacity, and latency before claiming independent replicas or reduced communication.

Placement changes where the required experts execute; it must not silently change which experts the model selects. Use real token routing when scoring output correctness. Frozen routing replay isolates data movement but is not an end-to-end quality test.

| Policy | Proposed experiment | Cost or risk to expose |
| --- | --- | --- |
| E01 — Static placement | Compare supported contiguous and round-robin assignments | Layer-specific skew and unequal expert bytes |
| E02 — Offline balanced placement | Use a training portion of the trace to balance bytes, expected compute, and communication; evaluate on held-out traffic | Overfitting the mapping to the measured trace |
| E03 — Topology-aware locality | Map expert owners and replicas to measured GPU/NIC/NUMA domains | Less network traffic can produce compute hotspots |
| E04 — Hot-expert replication | Replicate selected expert identities within a fixed extra-memory budget | Replica bytes compete with KV; account for initialization and updates |
| E05 — Online load balancing | Reassign/replicate experts at defined intervals with hysteresis | Migration bytes, transition pauses, stale epochs, and load oscillation |
| E06 — Tier-aware admission/prefetch | Compare LRU baseline with measured-reuse admission and deadline-aware prefetch | Wrong prefetch, read amplification, source congestion, and eviction churn |
| E07 — Tile streaming / ring buffering | Compare full-expert staging with bounded tiles only on a compatible execution path | Buffer size, dependency stalls, underflow, synchronization, and limited lookahead |

vLLM documents an expert load balancer and redundant-expert memory overhead. Placement controls and backend compatibility must be checked for the pinned version; these proposed tests are not claims that all strategies are already supported. [R9, R12]

## Preserve a fair experiment

Fix the model's top-k and router semantics. A topology-aware policy may select among identical expert replicas; it must not drop or substitute expert identities to keep traffic local. Record actual quality when precision or execution order changes.

Use natural skew, broad access, and shifting popularity. Separate a cold start from warm steady state. Score unique expert/shard loads per batch after coalescing; do not charge a full expert read once per token when the batch shares it.

Include no-prefetch and implemented-predictor controls. A perfect future-routing oracle is an upper bound only. Rings and prefetch are available design techniques for NVMe as well as HBF; compare both with the strongest practical software path.

Output: Which strategy helps, the workload region where it helps, extra memory/transfer costs, correctness results, and the runtime capability needed to support it.

# KV reuse and communication are separate from expert weights

KV tests remain in priority 1, but should not be folded into the expert-cache model. KV identity depends on the model, tokenizer, positions, precision, layout, and supported execution configuration. Retained state must be tenant-scoped and versioned. [S1, S2]

## Proposed KV comparisons

| Mode | What is fixed | What it tests |
| --- | --- | --- |
| K01 — Active KV | Supported attention semantics and prompt/output trace | Local working-memory occupancy and decode behavior |
| K02 — Prefix reuse | Identical reusable prefixes and model identity | GPU/host/tier hits versus prefill recomputation |
| K03 — Pause/resume | Known wait intervals and resume arrivals | Retain versus restore versus recompute, including demotion cost |
| K04 — P/D handoff | Same model and validated KV compatibility | Immediate worker-to-worker transfer, not long-term cache retention |

Do not classify old tokens as cold solely because of age. For full attention, old KV can still be read during active decoding. Sparse or approximate attention changes belong in a separately quality-validated study, not the initial storage comparison.

## Software roles

Use an existing KV connector/LMCache path where compatible. A MoE expert loader is a separate integration. Dynamo can support service routing and phase separation; Mooncake/NIXL are possible transfer/storage components in supported stacks. No experiment requires all of them, and their presence does not establish HBF support. [R15, R16]

## Keep three byte counters

Expert/weight bytes: startup, cold misses, prefetch, and migration.

Activation/collective bytes: TP/EP dispatch, combine, and pipeline exchange.

KV bytes: local reads, demotion, reuse restore, and P/D handoff.

They can contend on shared resources but should remain distinguishable in traces. A local HBF advantage may vanish at a remote-network boundary; model and measure that boundary explicitly.

Ring accounting: Distinguish command descriptors, completions, and data payload buffers. A ring can hide limited latency when there is sufficient lookahead and source rate. It cannot solve a sustained deficit in producer bandwidth. Record occupancy, underflow time, wasted prefetch, and the memory consumed by buffering.

# Software and Kubernetes: make each configuration usable

The common layer is the workload API and measurement contract, not a promise that the same CUDA binary, kernel, or driver runs on ATMOS.

| Layer | Initial implementation | ATMOS/HBF readiness requirement |
| --- | --- | --- |
| Serving API | One request/replay harness and result format | Supported generation, cancellation, streaming, errors, and model identity |
| Runtime/compiler | Pinned vLLM or other selected NVIDIA backend | Actual operator, precision, model-layout, and collective support; record CPU fallbacks |
| Data access | NVMe/GDS and CXL paths with explicit destination buffers | Versioned read/prefetch/completion/reset contract; separate expert and KV adapters |
| Node enablement | GPU Operator or qualified host-managed drivers | ATMOS driver, firmware, userspace SDK, inventory, diagnostics, and upgrade ownership |
| Resource allocation | Existing device plugin; DRA where supported and useful | Whole-card allocation first; separate HBF claims only if independently assignable and isolated |
| Operations | Health probes, telemetry, namespace policy, and deployment automation | Correct health reporting, tenant cleanup, restart/fallback, and supported lifecycle |

NVIDIA GPU Operator provides an existing lifecycle model, and Kubernetes DRA provides richer resource selection. Neither supplies an ATMOS driver or creates hardware isolation. Reuse actual team components before inventing a new operator or SDK. Pin the complete Kubernetes/runtime/driver compatibility matrix. [R13, R14]

## Deployment sequence

First reproduce the workload in a container on one node. Then deploy it through Kubernetes and compare with the direct-container control. Add topology constraints and multi-node lifecycle only after the single-node path works.

For multi-rank serving, arrange coordinated placement/startup and group recovery; a surviving Pod is not proof that the serving group remains valid. Separate the Kubernetes decision to place a Pod from the router decision to assign each request. Dynamic expert/prefix metadata belongs in a runtime directory, not thousands of rapidly changing node labels.

## Security and observable capability

Prefer unprivileged workload containers; confine privileged driver operations to controlled infrastructure components. Validate device access, DMA isolation, model/KV namespace boundaries, zeroization or cryptographic separation, and audit trails.

Never advertise an emulated device as providing real GPU-direct DMA. Its resource label and result metadata must state that it is emulated. Capacity requests do not guarantee bandwidth QoS unless the hardware/runtime enforces it.

# TokenSim and other models: role and limits

Keep modeling components separate so that a faster assumed NPU cannot hide a slow HBF path, or vice versa. The initial simulator is not a complete RTL, firmware, or instruction-level model of ATMOS. [S1, S2]

| Component | Planned use | Limit |
| --- | --- | --- |
| TokenSim + project wrapper | Request arrivals, scheduling, KV reuse, parallelism, and capacity/latency sensitivity | Only the audited and calibrated paths of the pinned revision are trusted |
| ATMOS-system model | NPU operator timing, usable working memory, scheduling, and supported communication | Needs engineering profiles; peak TOPS is not measured serving throughput |
| HBF media/path model | Capacity, transfer sizes, service distributions, queues, copies, contention, rings, and errors | Media/device targets do not establish physical end-to-end behavior |
| Real-payload tier emulator | Exercise a real application's data-access API with controlled delays, limits, and faults | Bounded by its real backing memory, interconnect, and compute; cannot manufacture faster physical transfers |
| QEMU CXL fixture — optional | Exercise device discovery, memory-region setup, and selected software paths | Functional emulation, not a measured CXL latency/bandwidth control [R18] |
| SimPy/rack model — later | Extend calibrated node behavior with queues, links, placement, and recovery | Requires a network-sharing model; no unbounded independent bandwidth per request [R19] |
| Engineering functional/RTL/FPGA environment — if available | Validate a specific internal interface, operator, or ordering rule | Scope and timing fidelity depend on that environment; not assumed to exist |

## Reconcile the older charter with current upstream

POC 01 describes a project wrapper whose KV far-tier mapping does not automatically validate MoE behavior. The public TokenSim README checked for this revision now lists TP/PP/DP/EP, MoE placement/routing metrics, and Mooncake-style offload. These are different statements. Audit the team's pinned code, reuse supported features, and add only missing behavior. In particular, validate per-layer distinct expert loads, cache eviction/prefetch, routing lookahead, and the actual ATMOS destination-memory path. [S2, R17]

TokenSim's documented default latency backend uses a Linux x86_64 Python 3.11 extension; confirm reproducibility and the backend's limitations before adopting it. [R17]

Key boundary: Replayed or synthetic outputs from a simulator are not model-quality evidence. Quality is established on an actual executing reference and later on actual ATMOS execution.

# Priority 2: RAG and vector retrieval

RAG is deliberately a separate track because it introduces a vector/index engine, retrieval policies, document I/O, and additional queueing. It should not hide an LLM bottleneck or make a retrieval improvement look like an ATMOS compute gain.

| Test mode | What runs | What is held fixed |
| --- | --- | --- |
| R0 — Prepared-context LLM | LLM with preselected context chunks | Exact prompt bytes and output settings; no timed retrieval. This remains priority 1. |
| R1 — Retrieval only | Query vector, ANN search, candidate-vector fetch, optional exact rescore, and payload lookup | Corpus, stored precision, filters, index parameters, quality target, and memory/cache budgets |
| R2 — Combined RAG | Retrieval followed by the already qualified LLM service | Model and retrieval identities; matched requests; separate retrieval and generation spans |

Begin R1 with one selected engine, not every vector product. Use an engine-native DRAM/NVMe control and, where relevant, a verified GPU path. HBF is a candidate only after its attachment and data API are defined. A CPU-native database does not automatically gain access to accelerator-local HBF.

Measure Recall@K or an agreed retrieval-quality target, completed queries/second, p95/p99, actual physical reads, cache reuse, candidate count, and bytes/query. Exact rescoring of retrieved candidates is not the same as exact nearest-neighbor search over the whole corpus.

For R2, first replay a fixed retrieved-chunk set across generation architectures. Then run live retrieval in a separately labeled service experiment. Report API wait, retrieval, prompt construction, prefill, decode, tier movement, tool replay, and result-write time without double-counting overlap.

## What remains out of the initial implementation

Index training, online embedding generation, graph databases, transactional database caches, and general OLAP are later workload packs. The same platform can host their tests, but LLM simulation results cannot be reused as evidence for their performance. Each needs an engine-specific access trace, correctness/quality rule, and measured control. [S2, S3]

Priority rule: Finish the selected LLM and hardware-path baseline before R1. Start R2 only after both generation and retrieval have independently understandable results.

# Action plan and collaboration

This proposed sequence replaces the original six-week, finance-specific execution sequence for this expanded POC. It is not a hardware delivery commitment. Work on OEM compatibility begins immediately, even while the harness runs in cloud.

| Period | Main work | Output / next decision |
| --- | --- | --- |
| Weeks 1–2 | Confirm card/interface requirements; contact Dell and one alternative; pin code/models; build LLM-only harness | Scope, candidate BOMs, capability gaps, and reproducible smoke test |
| Weeks 3–4 | Measure A0/A1 and available B; audit TokenSim; capture rank maps and topology; qualify a physical OEM fixture where accessible | Measured controls and OEM shortlist; explain unavailable paths |
| Weeks 5–8 | Run feasible TP/EP cells; add selected placement/KV tests; calibrate separate models; deploy Kubernetes single-/two-node controls | LLM fit/no-fit results, software requirements, and calibrated model limits |
| Weeks 9–12 | Repeat decision cells on OEM hardware; test lifecycle and two-node contention; start R1 only if priority-1 baselines are complete | OEM integration report and selected scale/RAG follow-on |
| After usable ATMOS/HBF arrives | Reconfigure qualified fixture, run hardware bring-up acceptance, repeat frozen tests, and recalibrate | Physical component and system evidence, each labeled separately |
| After two-node proof | Selected four/eight-node measurements and larger rack sensitivity studies | Deployment recommendation, recovery limits, and production BOM options |

If hardware or runtime readiness slips, continue useful software and modeled work, but do not compress physical qualification into an arbitrary date. If a path fails, record whether the problem is workload fit, immature software, platform compatibility, or insufficient evidence.

# People, equipment, and engineering access

The following is a proposed active-phase staffing plan, not a staffing commitment. Reuse the existing portfolio team and equipment. One person may cover more than one role, but the schedule must reflect the actual available effort.

| Capability | Suggested allocation | Responsibility |
| --- | --- | --- |
| Platform/solutions lead (Shailesh) | 1.0 FTE | Own configuration matrix, workload decisions, OEM coordination, and report |
| Inference/runtime engineer (Shailesh) | 1.0 FTE | Serving correctness, TP/EP, expert/KV instrumentation, and backend integration |
| Systems/performance engineer (Shailesh) | 1.0 FTE | PCIe/NUMA/GDS/CXL measurements, topology, bottlenecks, and physical controls |
| Simulation engineer(Shailesh) | 0.5–1.0 FTE | Capability audit, models, trace replay, and calibration |
| Cloud/Kubernetes engineer(Shailesh) | 0.25–0.5 FTE | Environments, images, deployment, security, and operating tests |
| Validation + architecture/firmware reviewers (Raj) | 0.5–1.0 FTE combined | Correctness, profiles, counters, failure tests, and readiness evidence |
| Retrieval/data engineer(Shailesh) | Add 0.5–1.0 FTE in priority 2 | Vector/RAG engine controls and quality |

OEM hardware/support contacts, lab technicians, security, and economics reviewers are shared dependencies. Do not count their effort twice across POCs.

## Equipment by stage

Now: CPU simulator host, approved source repository and artifact storage, temporary GPU capacity, real NVMe controls, and access to the team's versioned ATMOS profiles.

OEM stage: One suitable evaluation server, qualified RTX cards, swappable approved E3.S storage configurations, balanced host DIMMs, documented NICs, BMC access, and server energy measurement. Obtain a CXL evaluation fixture separately if necessary.

Multi-node stage: A second compatible server, appropriate switches/cables/optics, synchronized timestamps, and measurements of both host and accelerator transfers.

Physical ATMOS stage: Named board/silicon revisions, firmware, driver, SDK/compiler/runtime, diagnostic access, card power/cooling requirements, and engineering support during bring-up. HBF-only component proof and ATMOS-system proof may require different fixtures.

## Cost treatment

Retain separate estimates for cloud runtime, OEM rental/purchase, drives/NICs/CXL, power/cooling, physical prototypes, and engineering effort. Obtain quotes after the configuration shortlist. The earlier $1K–$3K POC-01 analysis allowance is not the budget for this expanded hardware program. [S2]

# Results that engineering, management, and customers can use

## One scorecard, with separate evidence categories

| View | Required output |
| --- | --- |
| LLM performance | Sustainable requests/s at fixed quality and latency; separate input/output throughput; TTFT, per-request TPOT and token-gap tails; queue growth |
| Data movement | Expert, KV, activation, and storage traffic; exposed stall; effective path rate; contention; copy and buffer accounting |
| OEM/platform fit | Exact supported or tested configuration, replacement procedure, physical limits, firmware/software dependencies, and unresolved qualification |
| Operations | Startup, group restart, degradation, isolation, firmware rollback, thermal behavior, and failure impact |
| Economics | Cost per accepted request, accepted requests/kWh, useful work per server/rack unit; full-system boundary and utilization |
| Model fidelity | Calibration cells, held-out error, uncertainty, and regions where predictions are not reliable |

## Required result package

Deliver a short recommendation plus a reproducible technical bundle: platform passport; compatibility/replacement matrix; path measurements; exact TP/EP rank maps; workload/model identity; placement policy; raw traces and histograms; model-error report; failure results; and source-labelled cost/energy assumptions.

Run at least five randomized repetitions of decision-critical measured cells, with enough observations and duration for meaningful tail estimates. Separate warm steady state from cold start. Do not use the best run or add stage p99 values as though they were an end-to-end p99. Report quality failures, timeouts, retries, and rejected requests alongside goodput. [S3]

## How the findings will be used

Engineering: Specific changes to paths, buffers, queues, operators, drivers, topology, cooling, or deployment.

Product/OEM: A practical target configuration, replacement limits, and an agreed qualification path.

Field/marketing: A workload-fit story backed by an exact configuration and evidence level. Simulated results describe the capability required; measured HBF component results describe that path; only real ATMOS-system tests support ATMOS-system performance claims.

Use continue / narrow / no-fit / blocked by readiness as distinct conclusions. A planning materiality threshold can be selected before testing—such as the earlier 20% improvement target—but a performance threshold never substitutes for quality, compatibility, or operational acceptance. [S1, S3]

Definition of done: The team can state which workload/topology fits, what limits it, which OEM configuration can host it, what can be replaced, which evidence is physical, and what remains before deployment.

# Use Case 1 :- Financial Service.

Financial-services relevance is represented through request-size, concurrency, burst, session and shared-prefix distributions. The serving experiment does not implement the surrounding business application.

| Serving profile | Financial interpretation | Input / output target | Primary SLO emphasis |
| --- | --- | --- | --- |
| Interactive short | Employee or service assistant | 2K-8K / 250-750 | TTFT and TPOT |
| Interactive long | Policy or case analysis | 16K-32K / 500-1,500 | Latency plus concurrency |
| Batch long-context | Compliance document reasoning | 32K-64K / 1K-2K | Useful throughput and cost |
| Very-long context | Investigation or research synthesis | 64K-128K / 1K-4K | Capacity and completion time |
| Session-resume proxy | Paused session resumes with cached state | Variable / 250-1K | Restore latency and KV policy |
| Shared-prefix proxy | Common instructions/policies across requests | 8K-32K shared / variable | Prefix hit rate and TTFT |

# Model portfolio and deployment hypotheses

| Model class | Purpose | Initial hypothesis |
| --- | --- | --- |
| Small dense bring-up | Validate the simulator, API, queue, scheduler and telemetry. | Resident execution should expose compute and scheduling behavior without capacity pressure. |
| 70B-class dense control | Test weight/KV separation and interactive serving. | RTX likely leads on latency; ATMOS must meet the SLO and show memory or cost value. |
| Qwen3-235B-A22B candidate | Primary large-MoE proof. | ATMOS may reduce capacity-driven sharding if full weights remain accessible and expert fetch is hidden. |
| DeepSeek-class follow-on | Stress a larger MoE and compressed-KV architecture. | Add only after Qwen routing, placement and HBF modeling are calibrated. |

| Dimension | Sweep | Reason |
| --- | --- | --- |
| Context | 8K, 16K, 32K; 64K and 128K after primary proof | Prefill, attention and KV pressure. |
| Output | 250, 500, 1,000, 2,000 tokens | Decode duration and KV growth. |
| Active sequences | 1, 8, 16, 32, 64, 128, then saturation | Batching, memory capacity and tail latency. |
| Arrival | Steady, burst, business-hour and saturation | Queue behavior and capacity knee. |
| Precision | BF16/FP16 control; approved FP8/INT4 | Weight size, quality and compute. |
| KV precision | BF16; FP8 if supported | LPDDR/GDDR capacity and bandwidth. |
| Prefix reuse | 0%, 25%, 50%, 75% | Cache value and TTFT. |
| Expert locality | Uniform, skewed, hot and churn | HBF/cache/EP behavior. |
| Placement | Resident, partially resident, tiered | Exposed data movement. |
| Parallelism | TP, EP, DP and supported hybrids | Communication and model residency. |
| Prefill/decode | Aggregated then disaggregated | Resource specialization and transfer cost. |

# Appendix B. Sources and revision notes

Public references ground the conventional controls and explain available interfaces. They do not confirm ATMOS support. Save a snapshot and exact software revision when using a reference in an experiment; pages labeled “latest,” “main,” or “dev” can change.

R1 — Dell PowerEdge XE7740 specifications. Product page. Accelerator and E3.S storage options; not ATMOS qualification.

R2 — Dell XE7740 service documentation. PSU/GPU configuration matrix. Supported populations and power dependencies.

R3 — NVIDIA RTX PRO 6000 Blackwell Server Edition. Product specifications. Selected control's memory and bandwidth.

R4 — Lenovo RTX PRO 6000 Server Edition guide. Product guide. PCIe, memory, power, and no-NVLink distinction.

R5 — Lenovo SR675 V3 / SR675i V3 guide. Product guide. E3.S/RTX inference configuration and fixed-component constraints.

R6 — SNIA SSD form factors. EDSFF/form-factor reference. Mechanical/interface framing; exact variant still needs OEM confirmation.

R7 — NVIDIA GDS design guide. Design guide. Direct data paths, staging, topology, and application relevance.

R8 — Linux PCI peer-to-peer DMA. Kernel documentation. Topology, resource, and support constraints.

R9 — vLLM Expert Parallel Deployment. EP guide. Layer-group semantics and expert load balancing.

R10 — vLLM Data Parallel Deployment. DP guide. Distinction between complete replicas and shared MoE execution.

R11 — TensorRT-LLM Expert Parallelism. Expert TP/EP concept. Historical documented hybrid semantics; verify the selected contemporary backend before implementing.

R12 — vLLM engine arguments. Configuration reference. Check placement controls and feature combinations in the pinned release.

R13 — NVIDIA GPU Operator overview. Operator documentation. Reference lifecycle components; not an ATMOS implementation.

R13b — GPUDirect with GPU Operator. RDMA and GDS guide. Supported deployment prerequisites and transfer validation.

R14 — Kubernetes Dynamic Resource Allocation. DRA concepts. Capability selection and allocation; validate release-specific features and drivers.

R15 — LMCache architecture. Architecture guide. KV reuse and tier integration; not a general expert-weight loader.

R16 — NVIDIA Dynamo disaggregated serving. Development architecture reference. Phase separation and KV-transfer boundaries; freeze a release for testing.

R17 — Public TokenSim repository. README and feature guides. Current advertised simulation capabilities and Linux/Python requirements; audit the project's pinned implementation separately.

R18 — QEMU CXL emulation. Device documentation. Optional functional software fixture, not physical-path performance proof.

R19 — SimPy. Documentation. Discrete-event framework; application/network fidelity is the model author's responsibility.

R20 — SNIA PCIe tutorial. Recording and slides. Server topology and PCIe foundations.

R21 — MIT accelerator tutorials. Tutorial recordings and materials. Dataflow, data reuse, and accelerator design fundamentals.